Smart Sprayer

Control Tablet — User & Installation Guide

Product code: ATAR-TAB

[ PLACEHOLDER: FIG-COVER ] Hero photo of ATAR Control Tablet mounted in a tractor cabin

Document language: English (master)

Revision: 0.1 — DRAFT  •  Date: June 2026

# Document information

| Document title | ATAR Smart Sprayer — Control Tablet User & Installation Guide |
| --- | --- |
| Product code | ATAR-TAB |
| Audience | Trained installers, dealer technicians, ATAR field service, operators |
| Document language | English (master). Translations: DE, SR (Latin) |
| Revision | 0.1 — DRAFT |
| Date | June 2026 |
| Prepared by | Atar Agtek d.o.o. |
| Approved by | Mirko Suka |

# Table of contents

# 1. About this guide

This guide describes how to mechanically mount, electrically connect, and operate the ATAR Smart Sprayer Control Tablet (product code ATAR-TAB) inside a tractor cabin. The tablet serves as the primary user interface (cabin terminal) for monitoring and controlling the ATAR Smart Sprayer system in real-time.

Read this entire document before starting installation or operation.

## 1.1 Symbols used

WARNING  Indicates a hazard that may cause injury or permanent equipment damage if ignored.

CAUTION  Indicates a condition that may cause incorrect operation or reduced performance.

NOTE  Provides additional information, tips, or context.

# 2. Safety

Follow all the safety rules below at all times. They protect both the operator and the equipment.

## 2.1 General safety rules

- Installation must be performed by trained personnel or experienced operators.

- Before starting installation, ensure the tractor engine is turned off and the ignition key is removed.

- Mount the tablet securely so it does not obstruct the operator's view of the field or access to tractor controls.

- Do not operate the tablet's touchscreen while driving at high speeds or in dangerous terrain.

## 2.2 Critical do-not rules

WARNING  Never use standard consumer USB-C cables or chargers. The ATAR-TAB requires the supplied rugged, threaded power and data cable to prevent loose connections and electrical damage due to tractor vibration.

WARNING  Never mount the tablet in a location where it could interfere with the deployment of cabin airbags or safety systems.

WARNING  Never expose the tablet's electrical connectors to water or chemicals when the cable is disconnected. Always fit the protective cap when not in use.

CAUTION  Do not over-tighten the RAM mounting arm. Tighten only until the tablet is held firmly in place.

# 3. Product specifications

All values refer to the ATAR Control Tablet (ATAR-TAB), built on the rugged V12r vehicle tablet platform.

## 3.1 Mechanical

| Platform / Model | Rugged Android Vehicle Tablet (V12r) |
| --- | --- |
| Display | High-brightness, sunlight-readable touchscreen |
| Ingress protection | IP67 (waterproof, dustproof, shockproof) |
| Housing material | Heavy-duty chemical-resistant composite |
| Mounting interface | Standard VESA / RAM mount compatible (1.5-inch ball) |

## 3.2 Electrical

| Input voltage | 12 V DC (via threaded cabin power harness) |
| --- | --- |
| Connector type | Rugged threaded circular connector (power + data) |
| Power consumption | Typical ~10 W, maximum ~15 W |

## 3.3 Connectivity

| Wired data | High-speed wired Ethernet (via threaded connector) |
| --- | --- |
| Wireless data | High-speed Wi-Fi (for wireless monitoring and control) |
| Operating system | Optimized ATAR OS (pre-loaded with Controller App) |

[ PLACEHOLDER: FIG-01 ] ATAR-TAB front, side, and back views showing threaded connector and RAM mount

Fig. 1 — ATAR Control Tablet (ATAR-TAB) — front, side, and back views.

# 4. Required tools and materials

- Screwdriver set (flat-head and Phillips)

- Wrench set (for RAM mounting base bolts)

- Wire stripper and crimp tool (for power connection)

- Multimeter (to verify voltage and polarity)

- Cable ties for routing

# 5. Choosing the mounting location in the cabin

The tablet must be mounted inside the tractor cabin in a location that is easily visible and reachable by the operator.

## 5.1 Location requirements

- Within easy reach and clear view of the operator when seated in the driving position.

- Does not obstruct the operator's forward visibility or view of the sprayer boom.

- Does not interfere with any tractor controls, steering wheel, or passenger seat.

- Securely attached to a rigid cabin frame member or a clean glass window (using a heavy-duty suction cup).

## 5.2 Orientation

NOTE  The tablet is designed to be operated in landscape orientation. Adjust the mounting arm angle to minimize screen glare from direct sunlight.

[ PLACEHOLDER: FIG-02 ] Recommended mounting locations in the tractor cabin

Fig. 2 — Recommended mounting locations in the tractor cabin.

# 6. Installation procedure

## 6.1 Mounting the RAM base

- Identify a secure mounting point on the cabin frame or window.

- Bolt the RAM ball base to the cabin frame using the supplied hardware, or attach the heavy-duty suction cup base firmly to a clean glass window.

## 6.2 Attaching the tablet

- Bolt the RAM ball plate to the back of the ATAR-TAB using the supplied VESA screws.

- Connect the RAM mounting arm between the cabin base ball and the tablet ball.

- Adjust the tablet to the desired viewing angle and tighten the RAM arm knob firmly by hand.

[ PLACEHOLDER: FIG-03 ] Exploded view of RAM mounting assembly

Fig. 3 — Exploded view of RAM mounting assembly.

# 7. Electrical and data connections

## 7.1 Power connection

- Route the power harness from the tractor's 12 V switched power source (e.g., auxiliary power plug or ignition switch) to the tablet location.

- Connect the positive (red) lead to the positive (+) terminal, and the negative (black) lead to the negative (-) terminal or ground. Fit the supplied in-line fuse close to the power source.

## 7.2 Threaded connector attachment

- Align the keyway of the rugged, threaded connector on the cable with the socket on the back of the tablet.

- Push the connector in firmly until seated.

- Turn the threaded locking ring clockwise by hand until tight. Do not use tools. This creates a waterproof, vibration-proof seal.

## 7.3 Wired data connection

- If using a wired connection, route the data cable from the ATAR Central Unit to the tablet.

- Connect the data line to the threaded connector on the back of the tablet.

## 7.4 Wi-Fi data connection

- If using a wireless connection, power on the tablet.

- Open the Wi-Fi settings on the ATAR Controller App.

- Select the secure Wi-Fi network of your ATAR Central Unit and enter the password (provided in your system documentation).

[ PLACEHOLDER: FIG-04 ] Threaded connector attachment and cable routing

Fig. 4 — Threaded connector attachment and cable routing.

# 8. Operation and Controller App

## 8.1 Powering on

- Press and hold the power button on the side of the tablet for 3 seconds.

- Wait for the ATAR OS to load. The ATAR Controller App will launch automatically.

## 8.2 Controller App interface

The Controller App provides a real-time overview of your smart sprayer system:

- Status indicators: Shows connection status of the Central Unit and all spray modules.

- Weed detection overlay: Displays real-time camera feeds with highlighted weed targets.

- Nozzle control: Allows manual override or automatic spot-spraying mode.

- Pressure monitor: Displays current working pressure (4–6 bar).

## 8.3 Running tests

Before starting field operations, run the following tests from the app:

- Air-leak test: Select 'Air-Leak Test' in the diagnostics menu. Confirm pressure remains stable.

- Nozzle test: Select 'Nozzle Test'. The app will cycle through and activate nozzles 1 to 4 sequentially to verify pneumatic valve operation.

[ PLACEHOLDER: FIG-05 ] ATAR Controller App main screen and diagnostics menu

Fig. 5 — ATAR Controller App main screen and diagnostics menu.

# 9. Maintenance and troubleshooting

## 9.1 Routine maintenance

- Clean the touchscreen regularly using a soft, dry microfiber cloth. Do not use abrasive cleaners or solvents.

- Inspect the threaded connector and cable regularly for wear, cracks, or loose locking rings.

- Check the RAM mounting arm tightness before each operation.

## 9.2 Troubleshooting

| Symptom | Likely cause | Action |
| --- | --- | --- |
| Tablet does not power on | 1) Blown in-line fuse 2) Loose threaded connector 3) Tractor power source inactive | 1) Replace the in-line fuse with the same rating. 2) Check and tighten the threaded connector. 3) Ensure the tractor's ignition or auxiliary power is active. |
| No connection to Central Unit | 1) Loose data cable (wired) 2) Wi-Fi disconnected (wireless) 3) Central Unit powered off | 1) Check the wired connection at both ends. 2) Reconnect to the Central Unit's Wi-Fi network in settings. 3) Verify the Central Unit is powered on and status LEDs are active. |
| Touchscreen unresponsive | 1) Dirt or moisture on screen 2) System frozen | 1) Clean the screen with a dry microfiber cloth. 2) Press and hold the power button for 10 seconds to force a restart. |

# 10. Do's and don'ts — quick reference

| DO | DO NOT |
| --- | --- |
| Hand-tighten the threaded connector locking ring until fully seated. | Never use tools (pliers, wrenches) to tighten the connector. |
| Mount the tablet securely using the supplied RAM mount. | Do not leave the tablet loose in the cabin — vibration will damage it. |
| Fit the protective cap on the connector when the cable is unplugged. | Never expose open connectors to moisture, dust, or chemicals. |
| Clean the screen only with a soft, dry microfiber cloth. | Never use solvents, alcohol, or abrasive materials to clean the screen. |

# 11. Appendix A — Quick reference card

| Model | ATAR Control Tablet (ATAR-TAB) |
| --- | --- |
| Ingress protection | IP67 (waterproof, dustproof, shockproof) |
| Power input | 12 V DC (via threaded cabin power harness) |
| Connector type | Rugged threaded circular connector (power + data) |
| Connectivity | Wired Ethernet & Wi-Fi |
| Mounting | VESA / RAM mount compatible (1.5-inch ball) |
| Operating OS | ATAR OS with pre-loaded Controller App |
| Maintenance | Clean screen with microfiber cloth, check RAM tightness |

# 12. Appendix B — Figure list

| Fig. cover | Hero photo of ATAR Control Tablet mounted in a tractor cabin |
| --- | --- |
| Fig. 1 | ATAR-TAB front, side, and back views showing connector |
| Fig. 2 | Recommended mounting locations in the tractor cabin |
| Fig. 3 | Exploded view of RAM mounting assembly |
| Fig. 4 | Threaded connector attachment and cable routing |
| Fig. 5 | ATAR Controller App main screen and diagnostics menu |